Bastionado de redes

# y sistemas.

U3_A1 Logging en Mikrotik con freeradius

Francisco Iván Ramírez Ortega

Curso 2023 - 2024

IES El Rincón

Curso de especialización en Ciberseguridad en entornos de las tecnologías de la información.

# Logging en Mikrotik con freeradius

# Introducción

MikroTik es una empresa que desarrolla hardware y software para redes de datos. Su producto más conocido es RouterOS, un sistema operativo de enrutamiento que convierte un PC estándar en un enrutador dedicado con todas las funciones. Además de RouterOS, MikroTik también fabrica hardware específico para routers, como routers, switches y dispositivos inalámbricos.

FreeRADIUS es un popular servidor de autenticación y autorización de código abierto. Es utilizado comúnmente en redes de área local (LAN) e infraestructuras de acceso remoto para proveer servicios de autenticación centralizados, como autenticación de usuarios de red, control de acceso a la red y contabilidad de uso de la red. FreeRADIUS es altamente configurable y puede integrarse con una variedad de sistemas de directorio y bases de datos para gestionar usuarios y sus privilegios de acceso. Es ampliamente utilizado en entornos empresariales, educativos y proveedores de servicios de Internet (ISP)..

Winbox es una aplicación de software con interfaz gráfica que se utiliza para administrar dispositivos MikroTik.

# Objetivo

Elaborar un pequeño informe con los pasos necesarios para iniciar sesión en una red manejada por un dispositivo Mikrotik con freeradius.

# Herramientas

- Winbox (Linux: snap install winbox)

ÍNDICE

Logging en Mikrotik con freeradius	2

Introducción	2

Objetivo	3

Herramientas	3

Práctica	4

Instalación	4

Configuración - Servidor	4

Cliente	4

Usuario	4

Configuración - Router	5

Configuración de la autenticación	5

Configuración de radius	5

Otros	7

Reseteo	7

Bibliografía	12

# Práctica

## Instalación

En una máquina Ubuntu:

sudo apt update sudo apt install freeradius

## Configuración - Servidor

### Cliente

En el archivo de configuración de los cliente, el router en el que vamos a permitir el acceso:

nano /etc/freeradius/clients.conf

El siguiente comando comprueba que la sintaxis de los archivos de configuración es correcta

sudo freeradius -XC

### Usuario

Editamos el fichero:

sudo nano /etc/freeradius/users

ivan Cleartext-Password := "pass"

## Configuración - Router

### Configuración de la autenticación

En Winbox

System->User (AAA)

### Configuración de radius

192.168.88.69 es la ip en la que se encuentra el servidor radius.

**pass

# Otros

## Reseteo

- Desenchufar el dispositivo

- Mantener pulsado el botón RES (reseteo)

- Enchufar el aparato y esperar al pitido.

Interfaz gráfica

System -> Reset Configuration

Interfaz de comandos

/system reset-configuration

Configuración por defecto

#¦ Welcome to RouterOS! #¦ 1) Set a strong router password in the System > Users menu #¦ 2) Upgrade the software in the System > Packages menu #¦ 3) Enable firewall on untrusted networks #¦ ----------------------------------------------------------------------------- #¦ RouterMode: #¦ * WAN port is protected by firewall and enabled DHCP client #¦ * Ethernet interfaces (except WAN port/s) are part of LAN bridge #¦ LAN Configuration: #¦ IP address 192.168.88.1/24 is set on bridge (LAN port) #¦ DHCP Server: enabled; #¦ DNS: enabled; #¦ WAN (gateway) Configuration: #¦ gateway: ether1 ; #¦ ip4 firewall: enabled; #¦ NAT: enabled; #¦ DHCP Client: enabled; :global defconfMode; :log info "Starting defconf script"; #------------------------------------------------------------------------------- # Apply configuration. # these commands are executed after installation or configuration reset #------------------------------------------------------------------------------- :if ($action = "apply") do={ # wait for interfaces :local count 0; :while ([/interface ethernet find] = "") do={ :if ($count = 30) do={ :log warning "DefConf: Unable to find ethernet interfaces"; /quit; } :delay 1s; :set count ($count +1); }; /interface list add name=WAN comment="defconf" /interface list add name=LAN comment="defconf" /interface bridge add name=bridge disabled=no auto-mac=yes protocol-mode=rstp comment=defconf; :local bMACIsSet 0; :foreach k in=[/interface find where !(slave=yes ¦¦ name="ether1" ¦¦ name~"bridge")] do={ :local tmpPortName [/interface get $k name]; :if ($bMACIsSet = 0) do={ :if ([/interface get $k type] = "ether") do={ /interface bridge set "bridge" auto-mac=no admin-mac=[/interface get $tmpPortName mac-address]; :set bMACIsSet 1; } } :if (([/interface get $k type] != "ppp-out") && ([/interface get $k type] != "lte")) do={ /interface bridge port add bridge=bridge interface=$tmpPortName comment=defconf; } } /ip pool add name="default-dhcp" ranges=192.168.88.10-192.168.88.254; /ip dhcp-server add name=defconf address-pool="default-dhcp" interface=bridge lease-time=10m disabled=no; /ip dhcp-server network add address=192.168.88.0/24 gateway=192.168.88.1 comment="defconf"; /ip address add address=192.168.88.1/24 interface=bridge comment="defconf"; /ip dns { set allow-remote-requests=yes static add name=router.lan address=192.168.88.1 comment=defconf } /ip dhcp-client add interface=ether1 disabled=no comment="defconf"; /interface list member add list=LAN interface=bridge comment="defconf" /interface list member add list=WAN interface=ether1 comment="defconf" /ip firewall nat add chain=srcnat out-interface-list=WAN ipsec-policy=out,none action=masquerade comment="defconf: masquerade" /ip firewall { filter add chain=input action=accept connection-state=established,related,untracked comment="defconf: accept established,related,untracked" filter add chain=input action=drop connection-state=invalid comment="defconf: drop invalid" filter add chain=input action=accept protocol=icmp comment="defconf: accept ICMP" filter add chain=input action=accept dst-address=127.0.0.1 comment="defconf: accept to local loopback (for CAPsMAN)" filter add chain=input action=drop in-interface-list=!LAN comment="defconf: drop all not coming from LAN" filter add chain=forward action=accept ipsec-policy=in,ipsec comment="defconf: accept in ipsec policy" filter add chain=forward action=accept ipsec-policy=out,ipsec comment="defconf: accept out ipsec policy" filter add chain=forward action=fasttrack-connection connection-state=established,related comment="defconf: fasttrack" filter add chain=forward action=accept connection-state=established,related,untracked comment="defconf: accept established,related, untracked" filter add chain=forward action=drop connection-state=invalid comment="defconf: drop invalid" filter add chain=forward action=drop connection-state=new connection-nat-state=!dstnat in-interface-list=WAN comment="defconf: drop all from WAN not DSTNATed" } /ip neighbor discovery-settings set discover-interface-list=LAN /tool mac-server set allowed-interface-list=LAN /tool mac-server mac-winbox set allowed-interface-list=LAN } #------------------------------------------------------------------------------- # Revert configuration. # these commands are executed if user requests to remove default configuration #------------------------------------------------------------------------------- :if ($action = "revert") do={ /user set admin password="" /system routerboard mode-button set enabled=no /system routerboard mode-button set on-event="" /system script remove [find comment~"defconf"] /ip firewall filter remove [find comment~"defconf"] /ip firewall nat remove [find comment~"defconf"] /interface list member remove [find comment~"defconf"] /interface detect-internet set detect-interface-list=none /interface detect-internet set lan-interface-list=none /interface detect-internet set wan-interface-list=none /interface detect-internet set internet-interface-list=none /interface list remove [find comment~"defconf"] /tool mac-server set allowed-interface-list=all /tool mac-server mac-winbox set allowed-interface-list=all /ip neighbor discovery-settings set discover-interface-list=!dynamic :local o [/ip dhcp-server network find comment="defconf"] :if ([:len $o] != 0) do={ /ip dhcp-server network remove $o } :local o [/ip dhcp-server find name="defconf" !disabled] :if ([:len $o] != 0) do={ /ip dhcp-server remove $o } /ip pool { :local o [find name="default-dhcp" ranges=192.168.88.10-192.168.88.254] :if ([:len $o] != 0) do={ remove $o } } :local o [/ip dhcp-client find comment="defconf"] :if ([:len $o] != 0) do={ /ip dhcp-client remove $o } /ip dns { set allow-remote-requests=no :local o [static find comment="defconf"] :if ([:len $o] != 0) do={ static remove $o } } /ip address { :local o [find comment="defconf"] :if ([:len $o] != 0) do={ remove $o } } :foreach iface in=[/interface ethernet find] do={ /interface ethernet set $iface name=[get $iface default-name] } /interface bridge port remove [find comment="defconf"] /interface bridge remove [find comment="defconf"] /interface bonding remove [find comment="defconf"] /interface wireless cap set enabled=no interfaces="" caps-man-addresses="" /caps-man manager set enabled=no /caps-man manager interface remove [find comment="defconf"] /caps-man manager interface set [ find default=yes ] forbid=no /caps-man provisioning remove [find comment="defconf"] /caps-man configuration remove [find comment="defconf"] } :log info Defconf_script_finished; :set defconfMode;

# Bibliografía

Comandos